Sukkur Institute of Business Administration University

Department of Computer Science

ESE-201: Digital Logic Design Lab

Lab # 11: Clocked SR Latch

Note: Submit this lab hand-out in the next lab with attached solved activities and exercises

Submission Profile

Name: Pranjal Mandhan					    Submission date (16/12/2023):

________________________________________________________________________________

Instructor Signature

Lab Learning Objectives:

Upon successful completion of this experiment, the student will be able:

- To implement and verify clocked RS latch operations using two NOR or two AND gates with clock & Module KL-33001

Lab Hardware and Software Required:

- 74LS02 IC

- 74LS08 IC

- Module KL-33001

- Breadboard

- Connecting Wires

Background Theory:

Clocked RS latch:

In RS latch we saw that the output was changing by changing both inputs. In sequential systems, the change of state in the latch is often requires to occur in synchronism with the clock pulse. Now the idea is that, third input is introduced i-e clock input (CK), which controls the output of the circuit. This is carried out by modifying the RS latch into the one of fig 11.1. In this case output will change only by changing the clock input.

This clock signal is generally a rectangular pulse train or a square wave. In most clocked latches the clock input is edge-triggered, which means that it is activated by a signal transition. This is indicated by the presence of a small triangle on the CK input. When clock changes from 0 to 1, this is called positive-going transition (PGT); when the clock changes from 1 to 0, this is called negative-going transition (NGT) of clock pulse. NGT is symbolized by presence of a small bubble on the clock input.

Fig 11.1: (a) Simplified block diagram of Clocked RS-latch (b) Clocked RS Latch

When the clock input is low (CK=0), the latch will not change its state and there would be no effect on the output Q. When clock goes high ((CK=1), then depending on the states of R and S inputs, the latch can set or reset or there can be no change in state.

Lab Examples:

Implementation of clocked RS latch using NOR & AND gates:

- Carry out the circuits of Fig 11.1 by using ICs of NOR and AND gates

- Make the circuit on breadboard present on the KL-33001 training kit

- Apply power supply to it

- Connect the inputs S and R of both circuits to the switches

- Connect the terminal of clock to the input CK of the latch

- Connect the outputs Q and Q of both circuits to the LEDs

- Analyze the circuit’s behaviour by carrying out different combinations with the switches and fill the observation tables

| Inputs |  |  | Outputs |  |
| --- | --- | --- | --- | --- |
| CK | S | R | Q | Q |
| 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 0 | 1 |

Table 11.1: Observation table of clocked SR latch

Lab Activities:

Change gated S-R latch to gated D latch and draw circuit diagram as well as truth table.

| Inputs |  | Outputs |  |
| --- | --- | --- | --- |
| CK | input | Q | Q |  |
| 1 | 1 | 1 | 0 |  |
| 1 | 0 | 0 | 1 |  |

Lab Exercises:

Implement Gated S-R and gated D- latch in Multisim and attach circuit diagram showing set and reset states.